# 📄 Secure Document Destruction Solution – Normal & High-Security

## Integrated Shredding Lines – 2,000 Tons / Year

### 1. Project Overview

This project is designed for bank-related archival document destruction, combining operational efficiency and high security compliance.

- Total volume: approx. 2,000 tons / year

  - Normal documents: 1,500 tons / year

  - High-security documents (P6–P7): 500 tons / year

- Operation assumption:

  - 200 working days / year

  - 6 operating hours / day

- Site condition:

  - Single centralized location

  - Area ≥ 500 m², height approx. 6 m

- Design objective:

  - Stable, long-term industrial operation

  - Modular & parallel production lines

  - Clear separation of security levels

### 2. Overall System Concept

We recommend an integrated solution with separated security levels and parallel production lines:

- Normal documents and high-security documents are processed on independent lines

- Multiple parallel lines ensure:

  - Operational stability

  - Maintenance flexibility

  - No single-point failure risk

- All systems are centrally managed within one facility

## 3. Line A – Normal Document Shredding (EU P2–P3)

Annual volume: approx. 1,500 tons

- Required throughput: 1.2–1.3 tons/hour

- Configuration: 2 parallel shredding lines

Each line includes:

- Primary shredder

  - Motor power: 15 kW

  - Output size: approx. 20 mm

- Secondary shredder

  - Motor power: 10 kW

  - Security level: EU P2–P3

- Single-line capacity: 600–700 kg/hour

Discharge & baling:

- Shredded paper is sent directly to the customer’s existing baler

- Large paper bales for easy handling and resale

- No additional baler investment required

## 4. Line B – High-Security Document Shredding (EU P6–P7)

Annual volume: approx. 500 tons

- Required throughput: approx. 400–420 kg/hour

- Configuration: 2 parallel high-security shredding lines

Each line includes:

- Conveyor feeding system

- Primary shredder

  - Motor power: 11 kW

- High-security fine shredder

  - Motor power: 22 kW

  - Output size: 1–3 mm particles

  - Security level: EU P6–P7

- Single-line capacity: 150–200 kg/hour

Final treatment:

- Shredded paper powder is processed by a hydraulic briquetting system

- Compressed into small cylindrical briquettes

- Ensures maximum security and reduced volume

## 5. Key Advantages of the Integrated Solution

- ✔ Parallel line design for high reliability and redundancy

- ✔ Clear separation of normal and high-security workflows

- ✔ Industrial-grade operation, not an oversized single machine

- ✔ Suitable for long-term operation in banking and archival environments

🌐 Contact Us 联系我们

Shanghai YUEDU Technology Co., Ltd. | 上海越都科技有限公司

📧 Email: yuedushredders@gmail.com | denghao8410@gmail.com 
📱 WhatsApp / WeChat: +86 159 0178 2779
🌐 Website: www.yuedushredders.com

@YUEDUShredders – Empowering Global Industries with Intelligent Destruction Systems.

# 🏦 银行档案文件安全销毁整体解决方案 - 普通文件 + 高保密等级文件

## 一、项目背景与目标

本方案面向银行、金融机构及专业档案销毁运营方，
用于集中处理 普通文件 + 高保密等级文件（P6–P7） 的长期、安全、合规销毁需求。

核心目标：

- ✔ 满足不同保密等级的合规要求

- ✔ 连续稳定运行，适合长期外包运营

## 二、整体方案架构

本方案采用 “两套工艺 + 一个整体系统” 的设计理念：

### 🔹 Line A｜普通文件粉碎 + 打包机

用于银行日常档案、普通文件集中处理

- 输送带自动进料

- 工业级粉碎机连续粉碎

- 粉碎后物料 直接进入打包机压块

- 压块成型，便于仓储与运输

👉 优势：

- 人工投入少

- 处理效率高

- 末端直接成块，降低物流成本

### 🔹 Line B｜高保密文件（EU P6–P7）粉碎成粉末 + 压块机

用于涉密文件、敏感档案、安全级别要求极高的文件

- 两条完全独立、结构相同的高保密粉碎线

- 每条线包含：

  - 一级粗碎

  - 二级细碎（达到EU P6–P7）,粉碎后为1~3mm粉末状

- 双线并行运行，保障连续性与冗余安全

- 粉碎后的纸粉进入液压式纸粉压块系统

- 压缩成小型圆柱状压块

## 三、每年产能

- 年处理总量：约 2,000 吨 / 年

  - 普通文件：约 1,500 吨/ 年

  - 高保密文件：约 500 吨/ 年

## 四、适用客户类型

- 银行档案中心

- 金融机构集中销毁中心

- 第三方专业保密销毁运营商

- 政府或金融外包销毁项目

📞 Contact Us

Johnny Deng, Managing Director

☎️  Phone: +86 159 0178 2779 (WhatsApp / WeChat / Facebook / Telegram)

📧Email: denghao8410@gmail.com

Shanghai YUEDU Technology Co., Ltd. (YUEDU Shredders)

🌐 Website(Global): www.yuedushredders.com (note: VPN required for China access, as the server is hosted by Google.)

📧 Email: yuedushredders@gmail.com / denghao8410@gmail.com

☎️  Sales Supervisor: +86 159 0178 2779  (WhatsApp / WeChat / Facebook / Telegram) ☎️  Administrative Supervisor: +86 137 6138 5116

☎️  Head Office Main Line: +86 21 519 000 20

📍 Company Headquarters: Building 12, No. 555 Chengyin Road, Baoshan City Industrial Park, Shanghai, China

与我们联系（中文普通话）

上海越都科技有限公司

📍公司总部：上海市宝山城市工业园区城银路555弄12号楼

☎️  中文电话: 138 1115 0898（同微信）, 137 1700 3125（同微信）, 188 1682 6112（同微信）

☎️  公司总机：021 - 519 000 20

☎️  行政总裁：137 6138 5116

☎️  投诉电话：159 0178 2779

📧  Email邮箱: denghao8410@gmail.com

🌐 Website(中文网站): www.91suizhiji.com

🌐 文档下载站： www.daxingsuizhiji.com